Guideline for building and setting-up a win32x64 PC computing system running NT-family Microsoft OS with PC GAMESS 6.x.

by Serge V. Kovrigin, IPMS & Kiev National University, ©2005

Table of Contents:

- Conventional vs. direct methods: tasks, where usage of conventional methods still reasonable.

- Building a PC for PC GAMESS conventional calculations: consideration of general aspects and desires.

- OS and PC GAMESS tuning procedure - achieving best performance.

*This how-to doesn’t claim to be an official and/or finished paper, neither has it come from the authors of GAMESS or PC GAMESS software; hereby, you may consider these materials as a set of recommendations, obtained as a result of usage of PC GAMESS by experienced users. This experience now is to your attention.

Why waste time for reading this?  Authors of this how-to achieved almost 100% efficiency (means CPU time/wall-clock time ratio) and three times as short calculation times compared to classical set-ups.

1. Conventional vs. direct methods: tasks, where their usage still reasonable.

PC GAMESS is being continuously developed and upgraded. As a result, in the last 6.5 build direct SCF code is almost as fast as conventional SCF code for basis-sets of medium complexity, and even faster in certain situations. Undoubtedly, future belongs to direct methods, due to their excellent scalability with parallel computing, higher reliability as a consequence of avoiding using HDD operations in most of the time and for same reason, lower costs for building a node in a cluster or workstation. However, at present time there are a lot of calculations, which can be performed much faster using conventional methods.

Typically, all these calculations are similar in several aspects: these are big systems (more than a thousand of base functions), with basis set used up to 6-31G**. Benefit from using techniques described below should be noticeable for any disk-intensive tasks, i.e. MP2, but it is only estimation and was not completely tested.

The root of all problems.

To tell it simpler, with conventional methods the more 2e integrals needed for a specific task, the wider bandwidth of HD-storage system should be. With increasing 2e integrals quantity not only storage capacity demands rapidly grows, but a number of integrals needed to be read/written per second increases. At some point, HDD storage system just cannot provide necessary throughput for PC GAMESS, and CPU has to wait for information to be delivered. This situation can be easily observed with a Task Manager on a P4-3GHz class PC running a 6-31G basis set job of about 800 basis functions in size, when CPU load drops to 60%.

On the other hand, PC GAMESS doesn’t operate HDD directly (as almost any other program) but through OS. With increasing number of read/write requests OS overload grows and more CPU time is distributed to serve those requests. As a rule, OS use its own cache to speed-up read/write operations (this referred to as ‘read-ahead’ and ‘delayed-write’ techniques), which in real slows down disk operations instead of speeding them up. This happens primarily because of OS assumption of random and repeated requests, while PC GAMESS most of the time uses sequential access to its files. There are some other reasons which lead to slowing everything down. Again, this can be observed with a Task Manager, with option ‘Show OS core time’ set. Red graph will show OS overhead, which for large tasks may almost be as high, as PC GAMESS CPU load.

These main two reasons make prerequisite the development of direct methods. But direct methods become ineffective when basis-sets used are not too complicated. As a result, we have something-like ‘death-zone’ for a range of tasks, which are too complicated for conventional methods, while still too simple for direct methods to be used. Following materials will show how to overcome this issue.

2. Building a PC for PC GAMESS conventional calculations.

With a wide variety of PC-based platforms and solutions on the market it is not easy to make a universal decision. The matter of fact is, however, that the final result and your satisfaction with your new workstation/node is not simply a function of money you spent! The general rule: “try”. You should test given configuration with your specific tasks. The best configuration will be the most reliable among the most fast ones. Please remember, even if you waste a month for a testing, you will save much more time in a real job.

Here we only point to several aspects you should keep in mind when building your “GAMESS PC”.

- Reliability. Your hardware must be reliable enough to operate for months without accidental reboots, errors, malfunctions. This demand becomes even stricter, if you plan to use your PC not for calculations only, but for office-work as well. Ideal, but expensive solution is to use server platforms. At the same time practice shows that high-end hardware parts for desktop PCs from brand manufacturers are good enough to be an optimal choice for the smaller price.

- Expandability. Pay special attention to upgrade opportunities of your hardware: first of all it applies to memory expandability. In your work, amount of memory will be one of the major factors of efficiency, so you must be able to add memory as needed. Upgrading processor also can boost performance to some extent.

- Harmonicity.  In conventional SCF calculations CPU power and HDD-storage system capacity should be balanced for your specific tasks. It is foolish to purchase high-end expensive processor and one-HDD storage-system (even if it is a SCSI disk), as more likely than work fast such system will work slower compared to not so fast processor and a 2-disk RAID (see below).

- Wisdom. Not all ‘brand new’ means perfection! Preferring new platforms and solutions you risk wasting your time and your money for glitches and errors. The optimum choice is chipsets of 3rd,4th… generation in a family (for Intel, for example, 845,865,875). Remember, any technology needs time to become reliable.

Below we consider some applied advices, but you should remember that all depends on your specific tasks, so you should be careful and make your tests independently.

Guidelines to building efficient and reliable PC-GAMESS computer

For all cases, amount of memory installed must overcome needed by your task plus OS demands and even 20-25% greater for a stable and efficient working.

Platform choice and discussion.

Intel platform:

-Processor with HTT support if office-working on computer is assumed.

-System bus speed: 533, 800 (makes no difference)

-Memory (266,333,400,500) (makes no difference), 2 channel mode.

-Motherboard: supporting RAID (I865,I875)

-RAID: built-in (best ones are ICH5R, ICH6R).

-HDDs: 2xSeagate SATAII 200Mb.

-Cooler: Zalman.

AMD platform:

-AMD64 2800+-3200+.

-System Bus speed (Hyperlink) – doesn’t matter, except for NVIDIA early chipsets (150)

-Memory speed– makes no difference.

-Motherboard: supporting RAID (VIA KT880Pro, SiS)

-RAID: built-in, on a bridge, VIA (for VIA chipsets), SiS, or third party manufacturers.

-HDDs: 2xSeagate SATAII 200Mb.

-Cooler: Zalman.

Note on platforms:

For conventional calculations the main limiting factor is HDD storage system throughput. Thus, both processor and platform should be chosen with keeping this detail in mind.

At present time, AMD64 3200+ and Intel P4 3200 are almost the same at performance in PC GAMESS. AMD64 makes SCF calculations faster to up to 20% than Intel does, while 2e integrals and 2e gradients are generally 10% slower. Taking into account lower prices for AMD64 platforms, author would recommend latter for conventional SCF calculations.

Note on Intel:

-HTT brings almost nothing to the speed of calculation, but it helps if you plan to use your computer for some office works while PC GAMESS is running.

-Higher practical limit for processor speed for 2 disks RAID-system is 3,2 GHz.

-Prescott core radiate much more heat, than Northwood core, thus it is potentially not as stable as Northwood is.  No performance boost compared to Northwood was marked.

-Best results in stability and performance were achieved on I875Chipset.

Note on AMD64:

-AMD64 2800+-3000+ is the optimum solution. AMD FXs at higher frequencies can provide up to 30% of performance boost (for several time greater prices). However, this performance stills generally unclaimed because of insufficient HDD storage-systems throughput even when RAID is used.

-Low-end AMD processors, like Sempron (Paris core) is simply a cut-off from AMD64 core: less cache (256 KB), 64-bit instructions are blocked, rating index differs from AMD64 processors (that means that Sempron 2800+ and AMD64 2800+ is NOT the same). However, when operating at equal core frequencies, these two cores are very close in performance, while considerably differs in prices. As far as 64-bit computing is not utilized in PC GAMESS by now (as well as in popular OSs), this processor may become your choice. Yet be aware that some of Paris dies maybe simply degraded and burned-out AMD64 ones, so their reliability may be not enough for building computer for PC GAMESS calculations. You should test everything carefully before making your final choice.

-At present time, VIA solutions seems to be the most stable and productive, with SiS next to them.

Note on Cooler:

-You are free to use one you like, but you should pay special attention to keeping your CPU as cool as possible. Best results achieved with pure copper coolers (like Zalman ones).

Note on RAID:

-There are several opportunities to make RAID in your system, thus enhancing your HDD storage system throughput and overall speed of calculations. These are:

- software RAID

- built-in RAID

- PCI-card RAID from third party manufacturers.

We will not here discuss 1), since it loads CPU, crashes if OS crashes and has some minor disadvantages. As for items 2) and 3), there is something one must know.

Basically, the best solution now is built-in RAID. On modern motherboards, RAID controller utilizes SATA or SATAII interfaces, and provides 2 SATA ports. Thus, only 2 disks RAID can be made. There are some exceptions on the market (like ICH6R on i925 chipset), which were advertised to have 4 disks RAID on-a-chip, so you should check if those are ‘in-flesh’ at the moment. Unfortunaly, due to revolutionary re-designing of Intel 925 chipset compared to preceding chipset family, this advantage of ICH6R sinks in other disadvantages. Also, there are a lot so-called 4-port RAID on the market, but at a closer look all these solutions are combined 2-ports on integrated controller, and 2-ports on add-on third-party chip, and this is the case, when 2+2 is not the same as 4. For above reasons we will discuss only 2 disks RAIDs.

Without going into details, ICH5R is simply the best among ALL RAID controllers (including PCI-card RAIDs). On I875 Chipset it provides up to 125Mb/sec reading and slightly less writing rates, which is simply doubling speed of single HDD (Seagate 200 SATAII), with CPU loading less than 5%.

Then comes built-in RAID controller on VIAKT880Pro for AMD64 platform. Results are slightly worse, but still high – 110Mb/sec.

Add-on chips from third-party manufacturers will make hardly more than 85-90 Mb/s and the worst result shows PCI RAID cards. For latter, even manufacturer name doesn’t play any role: simple 2-port FastTrack shows the same result as about-professional server 4-port Intel RAID card with its own In-Out Intel 386 processor and 64Mb of buffer memory! Results are as low as 65-70 Mb/sec.

The key for understanding this strange behavior is very simple: PC GAMESS in the main uses sequential disk access, thus any IO processors and buffers won’t help a bit. The only thing matter here is a bandwidth of IO channel between HDD storage-system and memory. In a built-in solutions, like ICH5R or VIA RAID, RAID controllers are linked to memory directly through high-speed bus, Intel hyper-link (266mb/sec) or AMD Hyper Transport (up to 1Gb/sec), which provides bandwidth enough wide for not to delay information being transferred from RAID controller. Thus, as soon as new drives are available, you can replace your old ones and achieve even greater performance rates.

When add-on chips are used on a motherboard to build a RAID, situation is much worse, because they are linked to PCI bus, which throughput is limited to (theoretically) 133 Mb/sec. In practice, it is even lower because of sharing PCI bus with other devices and transferring some subordinate service information. This situation is getting even worse if RAID controller located on an ordinary PCI card.

Summary: very simple. Only PCI express architecture can benefit from any add-on solutions. However, it is not stable enough, when speaking of Intel new chipset family. Of course, you can try using server platforms, where PCI 64 standard provides up to 512 Mb/sec capacities for PCI devices. Such platforms are a matter of necessity for dual-CPU solutions. As for today single-CPU computers, the choice is for built-in RAID controllers.

Note on Disks:

-You will need 2 of them. Ideal would be ones produced within one lot (with serial numbers differs in last letter/sign). This will guarantee their identity in main parameters: speed, access time, flash version and lifetime. Each of those is critical for RAID stability and performance.

-You choose manufacturers to your own taste. However, you should follow several recommendations:

- Do not choose low-end models.

- Pay attention to country of assembly - Malaysia and Singapore seems to be more reliable.

- From HDD specification you should worry about speed of sequential reading/writing and lifetime only.

- Remember that maximum performance in sequential operation achieved by disk with “most Gbytes per plate”.

- Buy large disks: suppose your task needs 80 GB, in that case you should purchase not less than 160 GB disk. In the beginning of disk reading/writing speed may differ by a factor of two! Only first 20-30% of disk’s volume is the most rapid.

- Use disks, which meet SATA-II specifications; NCQ support won’t do a thing, because it prevails only for random requests.

*Disks we a talking about are ones designed for desktop platforms. For server platforms, where SCSI disks are traditionally used, situation may differ to some extent, but not in general.

*Disks mentioned above meet all those requirements.

-If you plan working on computer, where PC GAMESS runs, you should also purchase a separate disk for OS and swapping file.

-COOLING. Your disks will work without a pause for weeks or months. They may become hot while their operation. Temperatures of above 50C are considered to be high and may lead to premature death of your HD. Optimum operational temperature is up to 40C. To achieve this level of temperatures, extra HDD cooling is the matter of necessity. You may locate powerful-enough fan (80 to 100 mm in diameter) in front of you HDDs, use water-cooling system, or (original solution) you may attach two BIG radiators (with radiative surface more than 200 cm^2) to each side of HDDs using thermal paste. Please note: using those ‘side fans’ covers’ doesn’t deliver from overheating!

-Mounting. Leave as much as possible space between lower and upper HDD! Locate system speaker – if it is too close to HDD, dismount it and move away!

Note on overclocking:

As always with overclocking, you do it on YOUR OWN RISC!

With both platforms (i875 and VIAKT880Pro) it is possible to overclock processors to about 10% without any after-effects (means, hardware damage or errors in PC GAMESS results).

Actually, everything is working fine even at 20% overclocking. But you must have some margins of safety, when playing with such things. Remember, your computers must be stable enough to work for months without errors. And overclocking is a straight way to violence that stability.

To minimize loose of reliability you should follow several items:

-Do not exceed ANY voltages in your system. Exceeding voltages will degrade your hardware. For some, maybe quite long period of time everything will work fine and then glitches will break out.

-High-quality cooling system is necessary. Temperature of processor should not exceed 45C (for Intel) and 50C (for AMD64), special memory cooling plates must be used, side, input and output coolers must be installed into system’s case.

-Overclocker’s special memory modules (for example, Corsair) with low timings are needed.

-Power supply must be of extra quality and power (400 Watt is a minimum).

Note on AMD64 platforms and VIA KT880Pro chipset:

Unlike Intel, increasing system bus frequency will increase SATA link operating frequency as well. Being normal at 150 GHz, SATA will become inoperable at 170-175 GHz, and you won’t boot until you change this back. Moreover, OS damage is possible. We do not recommend you to violence this limit. At the top of operational range (175GHz) SATA will work with errors – so PC GAMESS and RAID will periodically crash. To avoid this, avoid overclocking AMD64 on VIA chipset (you may use NVIDIA chipset instead), or DO NOT approach too close the deadline.

Again, for both platforms, 10% overclocking is a practical limit.

- OS and PC GAMESS tuning procedure - achieving best performance.

OS choice.

Among NT-family Microsoft OSs (NT5, Windows2000, Window2001 Server Edition, Windows XP HE, Windows XP Professional Edition, Windows 2003 Server Edition) you may choose one you prefer. Testing PC GAMESS with conventional methods showed very slightly different results in calculations speed. The only point you should notice is amount of free memory as it varies from OS to OS considerably. Also do not hesitate over installing latest updates and patches available for OS of your choice (with the only exception of SP2 for Windows XP). Do not forget to make snap-shots of your OS with special software or using build-in OS tools before any updates/patches applied, to have an opportunity of roll-back.

*Note on SP2 for Windows XP.

Although PC GAMESS was tested under Windows XP with SP2 installed and no problems or performance degradation were noticed, this Service Pack may cause a number of compatibility issues between other programs installed on computer where PC GAMESS runs, which in turn may lead to certain instability.

Setting up OS and software environment.

There are some general rules one can hold to:

- OS’s stability is inversely proportional to the number of programs installed.

- Do not use any of ‘optimizing-n-tuning software’: you may win a bit in performance, but you’ll loose a lot in stability.

- Disable all OS or third-party monitors, virus scanners and other resident software.

- Make sure that drivers for each device in  your system is certified by Microsoft Hardware Quality Labs; if certified drivers are not available, use the most stable among the newest ones; you’ll have to perform your own tests on stability.

HDD storage system setting-up.

Since you use RAID, you have to configure RAID at first. This is normally done thru special utility built-in a chip, which can be accepted thru a System BIOS or its own BIOS during boot procedure. Please read instructions on setting up your RAID in manual coming with your motherboard or add-on card.

In a few words, you’ll have to connect 2 equal SATA disks to 2 SATA port on the motherboard/add-on card, then enable RAID operation of controller in System BIOS, and reboot. While boot procedure, RAID BIOS will prompt to change RAID configuration. If you agree, a controller specific utility will be run. Using this utility, you may:

- Create RAID volume

- Delete RAID volume

- Configure RAID as RAID0 or as RAID1.

*Note on RAID.

After creation of RAID volume, OS (i.e. Windows) will see one RAID disk instead of two. Operation with these two disks as they were one is maintained by RAID driver. Please note that if you plan to use RAID volume in a role of boot or system disk, you’ll need floppy disk with that driver at initial stage of Windows installation procedure. Otherwise, if RAID is used as a working/storage disk only, you may install that driver later.

*Note on RAID level.

RAID0 and RAID1 are the basic RAID levels. Basically, RAID level 0 (aka RAID0) is used to combine disks into one to achieve better performance, and RAID1 is used to achieve better reliability by mirroring one disk to another. Typically RAID1 doesn’t give any benefit in sequential reading (though there are exceptions on the market), and never benefits in sequential writing. For this reason, we will not consider RAID1 configuration, and will focus on RAID0.

In simple, RAID0 uses two disks sequentially, striping them into logical blocks (so called stripes), somewhat like clusters, while RAID controller pretends to be an integral disk for a system. As a result, when data block is requested by OS, RAID controller divides it between two member disks, and each of them may retrieve its own portion of information simultaneously with the other; these two disks work in parallel and thus resulting throughput of RAID0 may be as high as doubled throughput of a member disk. However, this is the ideal scenario. With this approach, seeking times increase compared to single disk. Also, what happens if data block requested by OS is smaller, than the stripe size? Obviously, performance will degrade, and in classical RAID0 configurations (as default in every controllers stripe size =128K) we may come to a paradox, when PC GAMESS running with a single disk actually faster than one running on RAID0!

To avoid this, when formatting your RAID0 volume under OS, chose cluster size twice as large as stripe size you’ve chosen when building RAID. Best results in performance (almost doubling the throughput of a single disk) were achieved when used 16K for a stripe size and 32K for a cluster size in OS. Because of OS retrieves information from HDD storage system in units of clusters, and cluster is twice as large as the stripe is, EVERY request will be processed in parallel by two member disks. With this technique sequential reading rates of 125 MB per second are possible.

Note on File System.

You may use any FS you like, but in real there are two of them to make choice from: FAT32 and NTFS. FAT32 benefits from its simplicity and is several percents faster than NTFS is, speaking of PC GAMESS operation. We would recommend FAT32 for usage together with PC GAMESS, until your job is large enough to overcome 4GB FAT32 limit in file size. For example, PUNCH file may easily overcome this limit. In this case you’ll have to use NTFS.

Note on FS configuring.

You may set several System Registry keys to achieve even better performance, but be aware of changing other keys or using values other than specified here:

To do this, open ‘Start Menu’, then click ‘Run’, type ‘Regedit’ into a box and press ‘OK’. Registry editing tool will now open. Then, in the menu ‘Edit’ choose ‘Find’ and type ‘ReadAheadThreshold’ into a text box. In a minute, necessary registry section should be open. Please compare to this:

[HKEY_LOCAL_MACHINE\SYSTEM\ControlSet001\Control\FileSystem]

"NtfsDisable8dot3NameCreation"=dword:00000000

"Win31FileSystem"=dword:00000000

"Win95TruncatedExtensions"=dword:00000001

"NTFSDisableLastAccessUpdate"=dword:00000000

"ReadAheadThreshold"=hex:00,00,02,00

If what you see looks too different, use ‘Find Next’ command to search through registry.

Then, you should be able to change keys’ values. Please right-click on a key you want to change in the right section of the Registry editing tool. Context menu will appear, where 1st item will be ‘Change’. Please choose this item.

On next window opened, change value in a text box to your desired value, then press ‘OK’ and edit next value.

As a result, this registry section should look like this:

[HKEY_LOCAL_MACHINE\SYSTEM\ControlSet001\Control\FileSystem]

"NtfsDisable8dot3NameCreation"=dword:00000001

"Win31FileSystem"=dword:00000000

"Win95TruncatedExtensions"=dword:00000000

"NTFSDisableLastAccessUpdate"=dword:00000000

"ReadAheadThreshold"=hex:00,00,04,00

Alternate way of modifying these registry keys it to create new text file (using Notepad application) and copy previous section as is into this file; save then that file with any name, then find it in Explorer and change file extension from *.txt to *.reg. Then just double-click on that file will initiate merging of your system registry and updated registry section. Just answer ‘yes’ and your system registry will be updated.

*Note: on some web-sites you may find other keys from this section, which supposedly can affect performance (like ContigFileAlloSize key and others), but in real will degrade it. Be aware of tweaking FS and OS, as it is VERY easy to make OS unreliable.

Finally: after configuring your OS and software environment, and careful testing PC GAMESS running under tuned OS, make a snapshot of OS using OS utility (make a recovery point) or third-party utilities. If something goes wrong, you will be able to restore you tuned configuration in several minutes.

PC GAMESS location.

Best scenario possible is to locate PC GAMESS on separate RAID0 volume, where nothing, but PC GAMESS will be resident. If you want to make use of the rest of the RAID volume (suppose, your volume is as large as 400GB and you do not need all of them for PC GAMESS), create another logical partition within the RAID volume: in this way, your files will never be mixed with PC GAMESS work files, and no fragmentation will occur. If you do not plan using your computer for anything, but PC GAMESS, you may also create System disk within a RAID volume. In this case, however, you should make PC GAMESS partition first, then System partition and only then Storage partition. Otherwise, performance will degrade. All these manipulations are possible with special disk managing tools, like Partition Magic 8.0 or others.

*Note on swapping file:

While it is best solution to locate swapping file on a high-speed separate disk, if it is not needed for some reason you must locate it on PC GAMESS working disk to achieve best possible performance. This situation is normal when assembling node for a cluster, for example.

PC GAMESS specific settings.

Several useful keys for PC GAMESS itself will enhance performance considerably.

SYSTEM card:

- VOLSIZ=512. This key will force splitting all working files into pieces of 512 MB in size. If amount of memory on your computer greater than 512 MB, OS overhead will be considerably reduced and efficiency will dramatically rise.

- AIOBUF=8192 This key will set buffer size for reading/writing integrals to 8Mb. This value should be found out experimentally. Increases to some extent reading/writing performance of HDD storage system when OS cache is disabled (see below)

- IOFLGS (8) =16384 – forbids using OS caching. Use this keys with care as it may cause performance degrade as well as performance boost. Ideal for using on nodes, where nothing but PC GAMESS runs. On workstations, any competitive process may cause loose of synchronism between IO subsystem and CPU and thus will considerably reduce HDD storage system throughput.

SMP card:

- HTTFIX=.T.  – enables Hyper Threading support for Intel processors. This key must be present if HTT is enabled in hardware.

P2P card:

- P2P=.T. DLB=.t. MIXED=.T. Presence of this group enables Peer-to-Peer interface for network communication when nodes are joined into a cluster. With these keys using of different platforms (for example, Intel and AMD64) becomes possible. Also, due to dynamical balancing of nodes load, more of the task distributed to more powerful node, which avoids idling of faster nodes waiting for slower to finish their job. It becomes evident when in cluster there are AMD64 and Intel P4 nodes are mixed: AMD64 looses in speed at 2e integrals and 2e gradient evaluation, but overperforms Intel P4 during SCF calculation. To smooth this difference it is useful to engage dynamical loading technique.

And, finally, some minor advices:

- Assign lowest level of priority possible to PC GAMESS.  OS will operate then in the most effective way.

- Periodically check you PC GAMESS disk (RAID) for errors. Before any large job check everything out to be in normal, de-fragment PC GAMESS disk.

- If you get FSF fatal error you should consider reduce your overclocking rate or, if no overclocking is in effect, carefully test your hardware parts. You may check if there is okay with CRC controlling key in PC GAMESS. This will slow everything down (5-9%) but will give you assurance that your HDD storage system operates normally.

Summary: very simple. Carefully chosen hardware parts, understanding bottlenecks in conventional PC GAMESS calculations and workarounds for them, fine tuning of OS and software environment, not violating overclocking margins gives one possibility to assemble very reliable, fast and not expensive workstation/node for quantum chemistry works. Isn’t that what you wanted?